# Hardware Pricing

🔧 REQUIRED HARDWARE COMPONENTS (To Build & Install)

These are the core components needed to build per vehicle to enable remote monitoring, data collection, and smart intervention (e.g., stop vehicle):

| Component | Quantity | Estimated Cost (ZAR) | Purpose |
| --- | --- | --- | --- |
| Microcontroller (Arduino Un / ESP32) | 1 | R120 – R200 | Main logic controller (reads sensors, sends signals) |
| OBD-II Adapter (ELM327 / STN1110) | 1 | R150 – R300 | Reads car data (speed, RPM, etc.) from CAN bus |
| GPS Module (NEO-6M) | 1 | R90 – R150 | Gets car's real-time location |
| GSM/GPRS Module (SIM800L or A6) | 1 | R100 – R250 | Sends alerts via SMS/data; connects to backend |
| Relay Module (2-Channel / 4-Channel) | 1 | R30 – R60 | Controls kill switch to stop/start the car |
| MPU6050 (Accelerometer/Gyro) | 1 | R30 – R60 | Detects abnormal movement, acceleration, tampering |
| Voltage Regulator (Buck Converter) | 1 | R30 – R60 | Converts car’s 12V to 5V for Arduino & sensors |
| Power Protection Circuit | 1 | R40 – R80 | Protects components from voltage spikes from the car battery |
| Custom PCB / Breadboard | 1 | R50 – R100 | Compact layout for components (for production scaling) |
| Tamper-Proof Enclosure (Plastic or Metal) | 1 | R80 – R150 | Protects components and hides them in the car |
| Miscellaneous (wires, resistors, fuses, SIM card, data costs) | ---- | R100 – R200 | Extra supplies for installation |

🧾 ESTIMATED HARDWARE COST PER VEHICLE:

| Category | Estimated Cost Range |
| --- | --- |
| Minimum Build (ESP32, no custom PCB) | R850 – R1,200 |
| Reliable, clean build (with housing, protection, PCB) | R1,300 – R1,800 |

I can reduce costs further by ordering in bulk or custom designing a single PCB with all modules embedded.

🔄 OPTIONAL / PHASE 2 ADD-ONS

| Component | Purpose | Extra Cost (ZAR) |
| --- | --- | --- |
| Backup Battery / UPS | Keeps system running if thief disconnects car battery | R150 – R300 |
| Dashcam with storage | Records visuals during suspected theft | R300 – R700 |
| RFID tag reader | Authenticate driver before ignition | R100 – R200 |
| Bluetooth module | Detect proximity of owner’s phone (auto-auth) | R60 – R100 |

✅ Final Notes:

- For high-end cars (2020+), the system could instead plug into factory APIs (through manufacturer SDKs or telematics).

- For older cars, this DIY hardware setup becomes essential.

Arduino Uno VS ESP32:
Arduino is easier for beginners, but ESP32 has built-in Wi-Fi + Bluetooth, which makes it better for long-term scaling and security. ESP32 is recommend.